# Beginner Friendly Build Guide

## Smart Traffic Management System using ESP32

# 1. What This Project Does

This project is a small smart traffic signal system.

It uses:

- 1 ESP32 board

- 4 Ultrasonic sensors

- 4 LEDs

- Jumper wires

- Breadboard

The system checks which lane has a vehicle first.

If a vehicle is detected:

- That lane gets GREEN light

- Other lanes stay RED

- Minimum green time is 3 seconds

After the lane becomes empty:

- The system checks the next lane in queue

- If a vehicle is still waiting there, it becomes GREEN

- If no vehicle is there, it skips to the next lane

The ESP32 also creates a Wi‑Fi hotspot and opens a live dashboard.

# 2. Components Required

| Component | Quantity |
| --- | --- |
| ESP32 Development Board | 1 |
| HC‑SR04 Ultrasonic Sensor | 4 |
| LEDs | 4 |
| 220Ω Resistors | 4 |
| Breadboard | 1 |
| Jumper Wires | Many |
| USB Cable for ESP32 | 1 |
| Laptop with Arduino IDE | 1 |

# 3. Understanding the Parts

## ESP32

The ESP32 is the main brain of the project.

It:

- Reads the sensors

- Controls the LEDs

- Creates the Wi‑Fi dashboard

## Ultrasonic Sensor

This sensor measures distance.

It sends sound waves and waits for them to bounce back.

If a vehicle is close:

- Distance becomes small

- The lane is considered occupied

### Pins on HC‑SR04

| Pin | Meaning |
| --- | --- |
| VCC | Power |
| GND | Ground |
| TRIG | Sends signal |
| ECHO | Receives signal |

## LEDs

LEDs act like traffic signals.

| LED State | Meaning |
| --- | --- |
| ON | Green signal for that lane |
| OFF | Red signal |

# 4. Important Safety Notes

Before starting:

- Never connect wires while power is ON

- Double check all connections

- Do not connect 5V directly to ESP32 GPIO pins

- Keep the USB cable unplugged while wiring

# 5. Breadboard Basics

A breadboard helps connect components without soldering.

### Main sections:

- Red line → Positive power

- Blue line → Ground

- Middle holes → Component connections

All holes in one row are connected internally.

# 6. Pin Connections

## LED Connections

| Lane | ESP32 Pin | LED |
| --- | --- | --- |
| Lane 1 | GPIO 14 | LED 1 |
| Lane 2 | GPIO 27 | LED 2 |
| Lane 3 | GPIO 26 | LED 3 |
| Lane 4 | GPIO 25 | LED 4 |

### How to connect LEDs

- Long leg of LED → ESP32 pin through resistor

- Short leg → GND

Always use a resistor.

## Ultrasonic Sensor Connections

### Shared Trigger Pin

All sensors share the same TRIG pin.

| Sensor | TRIG Pin |
| --- | --- |
| All Sensors | GPIO 5 |

### Echo Pins

| Sensor | Echo Pin |
| --- | --- |
| Lane 1 Sensor | GPIO 18 |
| Lane 2 Sensor | GPIO 19 |
| Lane 3 Sensor | GPIO 21 |
| Lane 4 Sensor | GPIO 22 |

### Power Connections

| Sensor Pin | Connect To |
| --- | --- |
| VCC | 5V |
| GND | GND |

# 7. Step-by-Step Wiring Instructions

## Step 1 — Place ESP32

Put the ESP32 in the middle of the breadboard.

## Step 2 — Connect Power Rails

Connect:

- ESP32 5V → Breadboard positive rail

- ESP32 GND → Breadboard negative rail

## Step 3 — Connect LEDs

For each LED:

- Insert LED into breadboard

- Connect resistor to long leg

- Connect resistor to GPIO pin

- Connect short leg to GND

Repeat for all 4 LEDs.

## Step 4 — Connect Ultrasonic Sensors

For each sensor:

- Connect VCC → 5V

- Connect GND → GND

- Connect TRIG → GPIO 5

- Connect ECHO → Correct GPIO pin

## Step 5 — Final Check

Before powering ON:

Check:

- No loose wires

- No crossed power wires

- LEDs are facing correct direction

- Sensor pins are correct

# 8. Installing Arduino IDE

## Download Arduino IDE

Install from:
https://www.arduino.cc/en/software

# 9. Installing ESP32 Board

## Step 1

Open Arduino IDE.

## Step 2

Go to:

File → Preferences

## Step 3

Paste this URL in:

Additional Boards Manager URLs

https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

## Step 4

Go to:

Tools → Board → Boards Manager

Search:

ESP32

Install:

ESP32 by Espressif Systems

# 10. Uploading the Code

## Step 1

Connect ESP32 using USB cable.

## Step 2

Open the .ino project file.

## Step 3

Select board:

Tools → Board → ESP32 Dev Module

## Step 4

Select correct COM port.

## Step 5

Click Upload.

## If Upload Fails

Press and hold BOOT button while uploading.

Release after upload starts.

# 11. How the Queue System Works

The system works like a real smart traffic signal.

## Example

### Scenario:

- Vehicle arrives in Lane 2

- Vehicle arrives in Lane 4

- Vehicle arrives in Lane 1

### Queue becomes:

Lane 2 → Lane 4 → Lane 1

### Signal order:

- Lane 2 gets GREEN

- After empty, Lane 4 gets GREEN

- After empty, Lane 1 gets GREEN

The system skips empty lanes automatically.

# 12. Minimum Green Time

Each lane stays GREEN for at least:

3 seconds

Even if the vehicle leaves immediately.

This prevents rapid switching.

# 13. Dashboard Access

The ESP32 creates a Wi‑Fi hotspot.

## Step 1

Connect your phone/laptop to the ESP32 Wi‑Fi.

## Step 2

Open browser.

## Step 3

Type:

192.168.4.1

You will see the live dashboard.

# 14. Common Problems and Fixes

## LEDs Not Working

Check:

- LED direction

- Resistor connection

- GPIO pin number

## Sensor Not Detecting

Check:

- VCC and GND

- Echo pin connection

- Sensor facing direction

## ESP32 Not Uploading

Try:

- Different USB cable

- Different COM port

- Hold BOOT button

## Dashboard Not Opening

Check:

- Connected to ESP32 Wi‑Fi

- Correct IP address

# 15. Testing the Project

## Test Procedure

- Power ON system

- Place hand/object in front of sensor

- Check if corresponding LED turns ON

- Remove object

- Check if next queued lane becomes active

Repeat for all lanes.

# 16. Project Workflow Summary

Vehicle Detected

↓

Lane Added to Queue

↓

Current Lane Gets GREEN

↓

Minimum 3 Seconds

↓

Lane Clears

↓

Next Lane Checked

↓

Next GREEN Signal

# 17. Future Improvements

Possible upgrades:

- Add real traffic lights

- Add OLED display

- Add mobile app

- Add AI traffic prediction

- Add emergency vehicle priority

- Add solar power

# 18. Final Notes

This project is a beginner-friendly smart traffic management prototype.

Even if you are new to electronics:

- Follow the wiring carefully

- Double check connections

- Upload the code correctly

The project should work successfully.

Good luck and have fun building it.